Henderson County Inspections Department

Park Model Set-up Information

Code Requirements:

- It is not permissible to set up park models as a permanent dwelling unless it is labeled a HUD manufactured home or a NC Modular Validating Stamp.

- A single living unit that is primarily designed and completed on a single chassis, mounted on wheels, to provide temporary living quarters for recreational, camping. or seasonal use, is certified by the manufacturer as complying with all applicable requirements of ANSI Al19.5 and: (a) Has a gross trailer area not exceeding 400 square feet (37.15 square meters) in the setup mode or (b) If having a gross trailer area not exceeding 320 square feet (29.72 square meters) in the setup mode, has a width greater than 8.5 ft. (2.59 meters) in the transport mode.

- Towing tongue, wheels, and axles must remain on the unit at all times.

- Units cannot have any permanent plumbing or mechanical connections.

- Electrical supply requirements to be installed and inspected in accordance to Article 552. Park Model can not be hard wired per OSFM interpretation.

- Units may be temporary blocked up and anchored.

(See additional information attached from NC Dept. of Insurance and Appendix E of the 2019 Manufactured Home Code)